---

tags:

- c

- memory-layout

- stack-frame

- call-stack

- bss-data-text

- activation-records

date: 2026-08-27

day: 5

---

# Day 5: Memory Layout of a C Program, Stack Frames & Segments

---

## 1. Quick Reference & Cheat Sheet

### Virtual Memory Address Space Layout (Linux x86_64)

```text

High Memory (0x7FFF_FFFF_FFFF)

┌────────────────────────────────────────────────────────┐

│ Kernel Space (Protected, inaccessible to user code)    │

├────────────────────────────────────────────────────────┤

│ Stack Segment (Grows DOWNWARDS towards lower memory)   │

│   - Local variables, activation records, return addrs  │

│                          │                             │

│                          ▼                             │

├────────────────────────────────────────────────────────┤

│ Memory Mapping Segment (Shared libs, mmap files)       │

├────────────────────────────────────────────────────────┤

│                          ▲                             │

│                          │                             │

│ Heap Segment (Grows UPWARDS via brk / sbrk / mmap)     │

│   - Dynamic allocations (malloc, calloc, realloc)      │

├────────────────────────────────────────────────────────┤

│ BSS Segment (Uninitialized / 0-initialized globals)    │

├────────────────────────────────────────────────────────┤

│ Data Segment (Initialized non-zero globals & statics)  │

├────────────────────────────────────────────────────────┤

│ Text / Code Segment (.text, .rodata) [Read-Only, Exec] │

└────────────────────────────────────────────────────────┘

Low Memory (0x0000_0000_0000 - NULL pointer trap area)

```

### Memory Segments Summary

| Segment | Content | Permissions | Lifetime | Binary Footprint |

| :--- | :--- | :--- | :--- | :--- |

| **`.text`** | Machine code instructions | Read-Only, Executable (`r-x`) | Program runtime | Stored in ELF binary file |

| **`.rodata`**| String literals, `const` globals | Read-Only (`r--`) | Program runtime | Stored in ELF binary file |

| **`.data`** | Initialized non-zero global/static variables | Read/Write (`rw-`) | Program runtime | Stored in ELF binary file |

| **`.bss`** | Uninitialized or zero-initialized globals/statics | Read/Write (`rw-`) | Program runtime | **0 bytes** in binary (only records size in ELF header; kernel zeroes at load) |

| **Heap** | Dynamic memory (`malloc`, `calloc`) | Read/Write (`rw-`) | Until `free()` or exit | Allocated at runtime |

| **Stack** | Function frames, local variables, parameters | Read/Write (`rw-`) | Function execution | Allocated dynamically by CPU `$rsp` pointer |

---

## 2. In-Depth Theory & Low-Level Mechanics

### A. The Anatomy of a Stack Frame (Activation Record)

Each time a function is invoked in C, a new **Stack Frame** is pushed onto the stack. On x86_64 (System V AMD64 ABI):

1. **Parameter Passing:** The first 6 integer/pointer arguments are passed in registers (`rdi`, `rsi`, `rdx`, `rcx`, `r8`, `r9`). Any additional arguments are pushed onto the caller's stack in reverse order.

2. **`call` Instruction:** Pushes the 64-bit Return Address (instruction pointer `$rip` of the caller) onto the stack and jumps to the callee.

3. **Function Prologue:**

```nasm

push    rbp          ; Save caller's base pointer

mov     rbp, rsp     ; Establish new base pointer for current frame

sub     rsp, 32      ; Allocate 32 bytes on stack for local variables

```

4. **Function Epilogue:**

```nasm

mov     rsp, rbp     ; Deallocate local variables

pop     rbp          ; Restore caller's base pointer

ret                  ; Pop return address into $rip and jump back

```

```text

Stack Frame Layout:

High Addresses

┌──────────────────────────────┐

│ Function Arguments (7th+)    │

├──────────────────────────────┤

│ Return Address (Saved $rip)  │  <-- Pushed by 'call'

├──────────────────────────────┤

│ Saved Frame Pointer ($rbp)   │  <-- Base of current frame

├──────────────────────────────┤

│ Local Variables & Arrays     │

│ (e.g. char buf[16], int x)   │

├──────────────────────────────┤

│ Saved Registers (rbx, r12..) │

└──────────────────────────────┘  <-- Current Stack Pointer ($rsp)

Low Addresses

```

---

### B. Deep Recursion & Stack Overflow Dynamics

The OS allocates a fixed stack size per process (typically **8 MB** on Linux, checkable via `ulimit -s`).

* Each recursive call consumes stack space for return address, frame pointer, parameters, and local variables.

* When recursive depth exhausts available stack memory, the stack pointer `$rsp` collides with the guard page at the boundary of the stack segment, triggering an unrecoverable **Segmentation Fault (`SIGSEGV`)**.

#### Tail Call Optimization (TCO):

When a recursive call is in the **tail position** (i.e. the function simply returns the result of the recursive call without further arithmetic), modern compilers with `-O2` replace the `call` with a `jmp`, reusing the existing stack frame and executing in $O(1)$ stack space.

```c

// Non-tail recursive: Cannot be optimized without frame reuse

uint64_t factorial(uint64_t n) {

if (n <= 1) return 1;

return n * factorial(n - 1); // Must wait for result before multiplying

}

// Tail recursive: Optimizes to an in-place loop with 0 additional stack frames

uint64_t factorial_tail(uint64_t n, uint64_t accumulator) {

if (n <= 1) return accumulator;

return factorial_tail(n - 1, n * accumulator); // Tail call!

}

```

---

## 3. Thoughtful Mini-Project (~1 Hour Scope)

### Project Title: Runtime Virtual Memory Layout & Stack Dynamics Probe (`mem_layout_probe`)

#### Objective

Write a modular C introspection utility that maps its own memory layout at runtime by printing pointer addresses across different memory segments and dynamically measures stack frame consumption per recursive call.

#### Functional Requirements

1. **Segment Address Map:**

* Print addresses of functions in `.text`

* Print addresses of string literals in `.rodata`

* Print addresses of initialized globals in `.data`

* Print addresses of uninitialized statics in `.bss`

* Print addresses of dynamically allocated buffers in `Heap`

* Print addresses of stack variables in `Stack`

2. **Stack Growth & Frame Size Probe:**

* Implement a recursive function `probe_stack(int depth, uintptr_t prev_addr)` that tracks address differences between stack frames across 5 invocation levels.

* Dynamically calculate and display the direction of stack growth (Downwards towards lower memory vs Upwards).

* Calculate average stack frame overhead in bytes.

#### Complete Starter Code Implementation

```c

#include <stdio.h>

#include <stdlib.h>

#include <stdint.h>

#include <stdbool.h>

// Global Segment Variables

const char *rodata_str = "Read-Only String Literal (.rodata)";

int initialized_global = 0x12345678;        // .data

int uninitialized_global;                   // .bss

static int static_zero_global = 0;          // .bss

void dummy_function(void) {

// .text reference

}

void probe_stack_growth(int depth, uintptr_t prev_stack_addr) {

int local_var = depth;

uintptr_t current_stack_addr = (uintptr_t)&local_var;

printf("  [Frame %d] Local variable at: %p", depth, (void *)current_stack_addr);

if (prev_stack_addr != 0) {

ptrdiff_t diff = (ptrdiff_t)(current_stack_addr - prev_stack_addr);

printf(" | Delta: %+td bytes (%s)",

diff,

diff < 0 ? "Growing Downward" : "Growing Upward");

}

printf("\n");

if (depth < 4) {

probe_stack_growth(depth + 1, current_stack_addr);

}

}

int main(void) {

int main_local_stack = 42;

int *heap_alloc_1 = (int *)malloc(sizeof(int) * 64);

int *heap_alloc_2 = (int *)malloc(sizeof(int) * 1024);

if (!heap_alloc_1 || !heap_alloc_2) {

fprintf(stderr, "Heap allocation failed!\n");

return 1;

}

printf("========================================================\n");

printf("        C RUNTIME MEMORY SEGMENT INSPECTOR              \n");

printf("========================================================\n\n");

printf("[1] .text Segment (Code / Instructions):\n");

printf("    main() function address:          %p\n", (void *)(uintptr_t)main);

printf("    dummy_function() address:         %p\n\n", (void *)(uintptr_t)dummy_function);

printf("[2] .rodata Segment (Read-Only Data):\n");

printf("    String literal pointer:           %p\n\n", (void *)rodata_str);

printf("[3] .data Segment (Initialized Globals):\n");

printf("    initialized_global address:       %p\n\n", (void *)&initialized_global);

printf("[4] .bss Segment (Uninitialized / Zero Globals):\n");

printf("    uninitialized_global address:     %p\n", (void *)&uninitialized_global);

printf("    static_zero_global address:       %p\n\n", (void *)&static_zero_global);

printf("[5] Heap Segment (Dynamic Allocations):\n");

printf("    Heap chunk 1 (256 B):             %p\n", (void *)heap_alloc_1);

printf("    Heap chunk 2 (4096 B):            %p\n", (void *)heap_alloc_2);

printf("    Heap Growth Delta:                %+td bytes\n\n",

(ptrdiff_t)((uintptr_t)heap_alloc_2 - (uintptr_t)heap_alloc_1));

printf("[6] Stack Segment (Activation Records & Local Vars):\n");

printf("    main() local variable:            %p\n\n", (void *)&main_local_stack);

printf("--------------------------------------------------------\n");

printf("           STACK FRAME DYNAMICS & RECURSION PROBE       \n");

printf("--------------------------------------------------------\n");

probe_stack_growth(1, (uintptr_t)&main_local_stack);

free(heap_alloc_1);

free(heap_alloc_2);

return 0;

}

```

---

## 4. Error Handling & Defensive Programming Challenge

### Scenario: The Unbounded Stack Allocation (VLA) Crash Vulnerability

Examine the following file processing function written using Variable Length Arrays (VLAs):

```c

#include <stdio.h>

#include <string.h>

// BUGGY IMPLEMENTATION

void process_network_payload(const char *raw_data, size_t data_len) {

// VULNERABILITY: Variable Length Array (VLA) allocated on the STACK based on untrusted input length.

// If an attacker sends data_len = 10 * 1024 * 1024 (10 MB), the stack (8 MB max) instantly overflows,

// crashing the program without any opportunity to catch an error or return NULL!

char local_buffer[data_len + 1];

memcpy(local_buffer, raw_data, data_len);

local_buffer[data_len] = '\0';

printf("Processed payload safely: %s\n", local_buffer);

}

```

### Analysis of Vulnerabilities:

1. **Unchecked Stack Growth via VLAs:** Unlike `malloc()`, which returns `NULL` on failure, stack allocations adjust `$rsp` directly without runtime bounds checking. Allocating an unbounded VLA on the stack bypasses stack guards and immediately triggers a segmentation fault or security compromise.

2. **VLA Portability & Safety:** In ISO C11, Variable Length Arrays were made optional (`__STDC_NO_VLA__`) and are banned in the Linux Kernel due to stack exhaustion risks.

### Defensive Fix (Dual-Strategy Stack Cache with Heap Fallback):

```c

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <stdbool.h>

#define STACK_THRESHOLD_BYTES 1024U       // Safe fixed size for stack allocation

#define MAX_PAYLOAD_LIMIT (4U * 1024U * 1024U) // 4 MB hard safety ceiling

bool process_network_payload_safe(const char *raw_data, size_t data_len) {

if (raw_data == NULL || data_len == 0) {

return false;

}

if (data_len > MAX_PAYLOAD_LIMIT) {

fprintf(stderr, "Defensive Error: Payload size (%zu bytes) exceeds maximum limit (%u bytes)\n",

data_len, MAX_PAYLOAD_LIMIT);

return false;

}

char stack_buffer[STACK_THRESHOLD_BYTES];

char *working_buffer = NULL;

bool is_heap_allocated = false;

// Fast path: Use pre-allocated safe stack buffer if within threshold

if (data_len + 1 <= STACK_THRESHOLD_BYTES) {

working_buffer = stack_buffer;

} else {

// Slow path: Dynamically allocate on heap if size is large

working_buffer = (char *)malloc(data_len + 1);

if (working_buffer == NULL) {

fprintf(stderr, "Defensive Error: Failed to allocate %zu bytes on heap\n", data_len + 1);

return false;

}

is_heap_allocated = true;

}

memcpy(working_buffer, raw_data, data_len);

working_buffer[data_len] = '\0';

printf("Successfully processed %zu bytes of payload.\n", data_len);

// Guaranteed cleanup

if (is_heap_allocated) {

free(working_buffer);

working_buffer = NULL;

}

return true;

}

```